* The same steps for Switch &Router

الطريقة الأولى لتغيير الـ

Router-Host name :

Config                 Hostname

*الطريقة الثانية لتغيير الـ

Router-Hostname:

CLI           after get started press enter

Router > write enable

Router # write config                                 example

Router (config ) # write hostname  R1

Hostname     Done

How to create a password:

ــInside (R1 (config)#)          write line console 0        example

R1( config-line) #          write password  cisco123

R1 (config-line) #         write login

R1 (config-line)#          write exit

R1(config)#           write  exit

R1#

ــ To show the information :

Inside R#         write show run

ــ  To create a password for configuration :

Inside R1#        write config                                             example

R1 (config)#            write enable password 1234cisco

R1 #          write exit

ــ To store information from temporary settings to permanent settings:

Inside R1         write copy run startup-config

Press Enter

ــ  To encode password :

Inside R1         write config t

R1 (config )           write service password-encryption

R1 (config)           write  exit

R1 #        write show running

ــ To create message (message of the day (motd)) :              example

Inside R1(config)#          write banner motd “Hello world “

“ ”           important

R1 (config ) #       write exit

R1 #      write show run

Good luck *-*